| PV Panel DPP Workshop Build one mock Digital Product Passport in 15 minutes: start from the preloaded Chinese PV cell DPP, add Belgian assembly inputs, then read and export the result. Scenario: PV panel assembly in Belgium Linked upstream data: PV cell DPP from China Workshop output: one complete panel DPP package |  |  |  |
| --- | --- | --- | --- |
| 1 Product setup Set the product identity, reporting role, assembly location, reference flow, and common stage. | 2 Inventory Add each material or linked DPP row using the simplified quantities below. |  | 3 Results Calculate the GWP100 result, separate upstream cell impact from own assembly rows, then export. |

1  Product setup

| PRODUCT NAME PV panel | REPORTING / ASSEMBLY LOCATION Belgium (BE) |
| --- | --- |
| REFERENCE FLOW 1 assembled PV panel | LIFECYCLE STAGE Manufacturing / assembly |

Important interpretation: the preloaded cell DPP covers production in CN. The panel DPP is assembled in BE. China-to-Belgium transport is a separate optional information row and is not calculated unless transport factors are added.

2  Inventory rows to add

Use these workshop-friendly quantities. The database entry names are intentionally pragmatic proxies where the exact activity is not available in the precalculated lookup.

| Row | Quantity | Where | Database entry / note |
| --- | --- | --- | --- |
| Preloaded PV cell DPP | 0.94 m2 | CN | Linked DPP already loaded |
| Aluminium frame | 2.13 kg | BE | other non-ferrous metal products (market for) |
| Front glass | 8.81 kg | BE | glass and glass products - residual (market for) |
| Copper wiring | 103 g | BE | copper products (market for) |
| Silicone sealant | 122 g | BE | chemicals nec (market for) |
| Tin + lead solder | 13.6 g total | BE | lead, zinc and tin products (market for) |
| Plastic/polymer layers | 1.65 kg | BE | rubber and plastic products - residual |
| Process chemicals | 141 g | BE | chemicals nec (market for) |
| Packaging board | 763 g | BE | paper and paper products - residual |
| Assembly electricity | 14.0 kWh | BE | production of electricity (market for) |
| Optional logistics: CN to BE | custom tkm | CN -> BE | document now; calculate only when transport tkm factors are added |

3  Results to read after calculation

| Total panel impact Complete GWP100 result for the assembled panel. | Upstream linked DPP Impact imported from the Chinese PV cell passport. | Own assembly rows Impact from the Belgian materials and electricity added in the exercise. |
| --- | --- | --- |

Facilitator checks before export

| Location is explicit CN for linked cell DPP, BE for assembly rows. | Proxy choices are visible Participants can discuss where data is exact vs approximate. | Export is complete Non-calculated rows remain in the DPP as traceable information. |
| --- | --- | --- |